# Problem 1 – Total Heat Flux in a Flowing Fluid

## Problem Statement

Warm saline (modeled as water) flows steadily through a small, heated channel of length L=0.10 m. The flow is fully developed and one-dimensional in the x-direction. At the inlet (x=0) the temperature is T(0)=300 K, and at the outlet (x=L) the temperature is T(L)=310 K.

Assume:

- The temperature varies linearly with x between 0 and L

- The velocity is uniform and constant: vx=1.0×10−3 m/s

- Fluid properties (saline): ρ=1000 kg/m3, cp=4180 J/(kg⋅K), k=0.60 W/(m⋅K)

- The reference temperature is TR=T(0)=300 K

Determine, using Equation (2.27):

- Temperature profile and gradient

    - The linear expression for T(x) for 0≤x≤L

    - The temperature gradient dT/dx

- Heat flux at the midpoint x=L/2

    - T(L/2)

    - The convective contribution to the heat flux, ρvxcp(T−TR), at x=L/2

    - The conductive contribution to the heat flux, −k dT/dx

    - The total heat flux qxtot at x=L/2 and its direction

- Relative importance: at x=L/2, the percentage of the total heat flux carried by convection and by conduction

## Solution

Supplied information: L, T(0), T(L), the linear temperature profile, the uniform velocity vx, the properties ρ, cp, k, and the reference temperature TR.

Additional assumptions: steady state; the properties ρ, cp and k are constant over the 300–310 K range; the positive x-direction is the flow direction (inlet to outlet). The channel walls supply the heat that raises the fluid temperature along the channel. This wall heating does not enter the flux calculation at a single location, so no wall heat rate is needed.

### Part 1 – Temperature Profile and Gradient

Step 1. Write the general linear temperature profile (Part 1a). A linear profile passes through the two known end temperatures. Its slope is the temperature rise divided by the channel length:

T(x)=T(0)+T(L)−T(0)Lx

Step 2. Substitute the given end temperatures and channel length. This step evaluates the slope with units:

T(x)=300 K+310 K−300 K0.10 mx

T(x)=300 K+10 K0.10 mx

T(x)=300 K+100 K/mx  0≤x≤0.10 m

Check: T(0)=300 K and T(0.10 m)=300 K+(100 K/m)(0.10 m)=310 K. Both boundary values are recovered.

Step 3. Differentiate the profile to obtain the temperature gradient (Part 1b). The constant term has zero derivative, so the gradient is the slope of the line:

dTdx=ddx300 K+100 K/mx=0+100 K/m

dTdx=100 K/m

The gradient is positive and uniform: temperature increases in the +x (flow) direction everywhere in the channel.

### Part 2 – Heat Flux at the Midpoint x=L/2

Step 4. Locate the midpoint. This step gives the position at which all fluxes are evaluated:

x=L2=0.10 m2=0.05 m

Step 5. Evaluate the midpoint temperature (Part 2a). Substitute x=0.05 m into the profile from Step 2:

T​L2=300 K+100 K/m0.05 m=300 K+5 K

T​L2=305 K

Step 6. Write the convective heat flux (Part 2b). The convective heat flux is the thermal energy, measured relative to TR, carried by the bulk motion of the fluid. It is given by Equation (2.23):

qxconv=ρvxcpT−TR

Step 7. Compute the temperature excess relative to the reference temperature.

T−TR=305 K−300 K=5 K

Step 8. Compute the mass flux ρvx. This is the convective mass flux of Equation (2.20), the mass crossing a unit area per unit time:

ρvx=1000 kgm31.0×10−3 ms=1.0 kgm2⋅s

Step 9. Multiply by the specific heat. Use 1 J/s=1 W:

ρvxcp=1.0 kgm2⋅s4180 Jkg⋅K=4180 Jm2⋅s⋅K=4180 Wm2⋅K

Step 10. Evaluate the convective heat flux.

qxconv=4180 Wm2⋅K5 K

qxconv=20,900 W/m2 (in the +x direction, with the flow)

The sign is positive because vx>0 and T>TR: the warm fluid carries energy downstream.

Step 11. Write and evaluate the conductive heat flux (Part 2c). The conductive (molecular) heat flux follows Fourier’s law, Equation (2.9):

qx=−kdTdx

Substitute k and the gradient from Step 3:

qx=−0.60 Wm⋅K100 Km=−60 Wm2

qx=−60 W/m2 (in the −x direction, toward the colder inlet)

The negative sign means heat conducts down the temperature gradient, from the warmer outlet toward the cooler inlet, opposite to the flow.

Step 12. Combine the two contributions to obtain the total heat flux (Part 2d). The total heat flux is the sum of the convective and conductive fluxes, Equation (2.27):

qxtot=ρvxcpT−TR+qx=ρvxcpT−TR−kdTdx

Substitute the results of Steps 10 and 11:

qxtot​L2=20,900 Wm2+−60 Wm2=20,840 Wm2

qxtot​L2=20,840 W/m2≈2.08×104 W/m2 (in the +x direction, downstream)

The positive total means that the net transport of thermal energy at the midpoint is downstream with the flow. Conduction opposes it only slightly.

### Part 3 – Relative Importance of Convection and Conduction

Step 13. Form the sum of magnitudes used for comparison. The two contributions have opposite signs. Comparing each with the net total 20,840 W/m2 would give a convective share above 100% and a negative conductive share. The shares are therefore based on the sum of the magnitudes of the two mechanisms:

qxconv+qx=20,900 Wm2+60 Wm2=20,960 Wm2

Step 14. Compute the convective fraction.

Fractionconv=qxconvqxconv+qx=20,900 W/m220,960 W/m2=0.9971

Step 15. Compute the conductive fraction.

Fractioncond=qxqxconv+qx=60 W/m220,960 W/m2=0.0029

Check: 0.9971+0.0029=1.0000

Convection≈99.7%  Conduction≈0.3%

For comparison, relative to the net total flux, convection is 20,900/20,840=100.3% and conduction is −60/20,840=−0.3%. The conduction term reduces the net downstream flux by 0.3%. Either way, convection dominates.

Step 16. Check units, signs and physical meaning.

- Units: (kg/m3)(m/s)(J/(kg⋅K))(K)=J/(m2⋅s)=W/m2 for the convective term, and (W/(m⋅K))(K/m)=W/m2 for the conductive term. Both terms of Equation (2.27) have the units of heat flux.

- Signs: vx>0 and T>TR give positive convection. dT/dx>0 gives negative conduction, pointing down the temperature gradient.

- Physical meaning: a Péclet-type ratio confirms the dominance of convection:

ρvxcpT−TRk dT/dx=20,900 W/m260 W/m2≈348

Even at the slow speed of 1 mm/s, convection carries about 350 times more thermal energy than conduction, because water’s volumetric heat capacity ρcp is large compared with its conductivity k.

- Note on the reference temperature: the convective term, and so the percentages in Part 3, depend on the chosen TR. The conductive term does not. Choosing TR=T(0) measures energy relative to the inlet state, as the problem specifies.

- Check against the source solution: the given numerical results are correct. The source’s Part 3 line “Convective fraction:” is left incomplete before the magnitude-based calculation; that calculation is completed in Steps 13–15.