# Example 6.2.6.2 — Newtonian Flow Through an Inclined Channel with a Pressure Gradient

Question

Many devices used in biology and medicine, such as parallel-plate membrane blood oxygenators and dialysis units, force fluid through the narrow gap between parallel plates. Replace the free surface of the falling-film problem with a second stationary wall. The channel has height h, width W and length L, and its axis makes an angle α with the horizontal. A pressure P0 is imposed at the inlet (x=0, y=0) and a pressure PL at the outlet (x=L, y=0).

Predict the flow of a Newtonian fluid through the channel under the pressure difference P0−PL. Determine the pressure distribution, velocity profile, maximum velocity, volumetric flow rate and flow resistance.

Given: channel height h, width W, length L and inclination α; inlet and outlet pressures P0 and PL; fluid density ρ and viscosity μ. No numerical values are given, so the solution stays symbolic.

Coordinates: x points down the channel axis, and y is measured from the lower wall (y=0) to the upper wall (y=h). Gravity has a component gsinα in the +x-direction and a component gcosα in the −y-direction.

Additional assumptions: steady, laminar, fully developed parallel flow, vx=vx(y), with entrance effects neglected; incompressible Newtonian fluid with constant properties; no slip at both walls, from Equation (4.5).

Define the imposed pressure difference

ΔP=P0−PL

## 1 Apply the momentum balance in the x-direction

The shell and mass balance are the same as for the falling film, so vx depends only on y. The convective momentum terms cancel, as in Equations (6.17) and (6.19). Unlike the film, the pressure forces on the two ends of the shell do not cancel, because the pressure is imposed. The x-momentum balance on a shell of size Δx Δy and width W is

0=τyxy−τyxy+ΔyWΔx+Px−Px+ΔxWΔy+ρgsinαWΔx Δy

This is Equation (6.20) with the pressure term added; the slides do not number this form. Divide by the shell volume WΔx Δy and let Δx and Δy approach zero

0=−∂τyx∂y−∂P∂x+ρgsinα

In parallel flow, τyx depends only on y, so its derivative is an ordinary derivative

dτyxdy=−∂P∂x+ρgsinα

## 2 Find the pressure distribution

A momentum balance in the y-direction contains no shear or convective terms, because vy=0. The pressure force balances the weight component ρgcosα, which acts in the −y-direction

−∂P∂y−ρgcosα=0

Integrate with respect to y, with an arbitrary function f(x) as the constant of integration

P(x,y)=−ρgcosαy+f(x)  ∂P∂x=dfdx

Substitute into the x-momentum equation

dτyxdy=−dfdx+ρgsinα

The left side depends only on y and the right side only on x. Both must therefore equal the same constant, C1

dτyxdy=C1  −dfdx+ρgsinα=C1

Integrate the second equation for f(x)

f(x)=ρgsinα−C1x+C2

P(x,y)=−ρgcosαy+ρgsinα−C1x+C2

Apply the inlet pressure, P(0,0)=P0

P0=C2

Apply the outlet pressure, P(L,0)=PL

PL=ρgsinα−C1L+P0

Solve for C1

C1=P0−PLL+ρgsinα=ΔPL+ρgsinα

Substitute C1 and C2

P(x,y)−P0=−ρgcosαy−ΔPLx

At any height y, the pressure falls linearly along the channel, so the axial pressure gradient, ∂P/∂x=−ΔP/L, is the same everywhere. Across the channel, the pressure varies hydrostatically with the weight of the fluid above.

## 3 Find the shear-stress and velocity profiles

Integrate dτyx/dy=C1 and introduce the Newtonian constitutive relation. Using Equation (6.26)

τyx=−μdvxdy=ΔPL+ρgsinαy+C3

Divide by −μ and integrate again

vx=−1μΔPL+ρgsinαy22−C3μy+C4

Apply no slip at the lower wall, vx(0)=0

0=C4

Apply no slip at the upper wall, vx(h)=0

0=−1μΔPL+ρgsinαh22−C3μh  C3=−ΔPL+ρgsinαh2

Substitute C3 and C4, then factor out h2

vx=12μΔPL+ρgsinαhy−y2

vx=h22μΔPL+ρgsinαyh−y2h2

The shear stress follows by substituting C3

τyx=ΔPL+ρgsinαy−h2

The shear stress is zero on the centerline, y=h/2, and largest in magnitude at the two walls.

The velocity profile is parabolic and symmetric about the centerline (Fig. 6.11). Pressure and gravity drive the flow together through the combined term ΔP/L+ρgsinα.

Parabolic velocity profile between parallel plates

## 4 Find the maximum velocity

By symmetry, the velocity is greatest on the centerline, where dvx/dy=0. Substitute y=h/2

vmax=h22μΔPL+ρgsinα12−14

vmax=h28μΔPL+ρgsinα at y=h2

The profile can then be written as vx/vmax=4y/h1−y/h, the curve plotted in Fig. 6.11.

## 5 Calculate the volumetric flow rate

Integrate the velocity over the channel cross-section

QV=0hvx W dy

Change to ξ=y/h, dy=h dξ

QV=Wh32μΔPL+ρgsinα01ξ−ξ2dξ

Integrate

01ξ−ξ2dξ=ξ22−ξ3301=12−13=16

Multiply by the prefactor and combine over the common denominator L

QV=Wh312μΔPL+ρgsinα=Wh312μLΔP+ρgLsinα

The average velocity is ⟨vx⟩=QV/Wh=23vmax.

## 6 Interpret the result and find the flow resistance

Dependence on geometry and properties. The flow rate is proportional to the width W and inversely proportional to the viscosity μ. It varies with the cube of the channel height h, so small changes in the gap produce large changes in flow. For a fixed pressure difference, the pressure contribution varies as 1/L, because the driving gradient is ΔP/L. The gravity contribution does not depend on L.

Direction of flow. The flow is zero when the pressure and gravity terms cancel

ΔP+ρgLsinα=0  ΔP=−ρgLsinα

The flow is down the channel (positive) when ΔP > −ρgL sin α and up the channel (negative) when ΔP < −ρgL sin α. It is zero when the outlet pressure exceeds the inlet pressure by exactly ρgL sin α, so that the adverse pressure difference just balances gravity. With a horizontal channel (α = 0) and no imposed pressure difference, there is no flow.

Flow resistance. By analogy with Equation (6.53), the resistance is the total driving pressure divided by the flow rate

ℜf=ΔP+ρgLsinαQV

ℜf=12μLWh3

The resistance depends only on the fluid viscosity and the channel geometry. Every fluid element feels the same axial pressure gradient, even though the pressure itself depends on both x and y.

Check:

- No-slip conditions: vx(0)=vx(h)=0 from the y/h−y2/h2 factor.

- Units of QV: mm3Pa/Pa⋅s⋅m=m3/s. Both driving terms, ΔP and ρgLsinα, have units of pressure.

- Units of ℜf: Pa⋅sm/m4=Pa⋅s/m3, the same as the Poiseuille resistance of Equation (6.53).

- Link to the falling film: with ΔP=0, a channel of height H carries QV=ρgWH3sinα/12μ. A falling film of thickness H/2 carries, from Equation (6.31), ρgWH/23sinα/3μ=ρgWH3sinα/24μ. The channel therefore carries exactly twice the film flow. By symmetry, the zero-shear centerline of the channel plays the role of the film’s free surface, and each half of the channel matches the film profile of Equation (6.28) with h=H/2.

Check against the source solution: the pressure distribution, velocity profile, maximum velocity, flow rate and resistance are correct. Three corrections:

- As transcribed, the source states that flow is positive if ΔP>ρgLsinα and negative if ΔP<ρgLsinα. The threshold is −ρgLsinα, consistent with its own zero-flow condition ΔP=−ρgLsinα.

- The source says the flow is “directly proportional to the length” of the channel. For a fixed ΔP, the pressure-driven part of the flow is inversely proportional to L, and the gravity-driven part does not depend on L.

- The source labels the maximum velocity “(6.32a)”. That number does not appear on the slides, where Equation (6.32) is the falling-film thickness, so it is not used here.