# Problem 15 — Maximum Shear Stress in an Arteriole

Question

Blood with a hematocrit of 45% flows through a horizontal arteriole of diameter D=20 μm and length L=2 mm. The pressure is P0=60 mmHg at the entrance and PL=20 mmHg at the exit. Calculate the maximum shear stress, and state where it occurs.

Cylindrical fluid shell of radius r, thickness Δr and length Δz inside a tube of radius R

Given: D=20 μm, L=2 mm, P0=60 mmHg, PL=20 mmHg, hematocrit 45%; the arteriole is horizontal.

Additional assumptions: steady, laminar, fully developed flow in a straight, rigid tube of constant circular cross-section; the flow is one-dimensional, vz=vz(r); gravity does not act along the horizontal axis. No constitutive relation for blood is needed, because the shear-stress distribution follows from the momentum balance alone.

## 1 Convert the given values

Tube radius

R=D2=20 μm21 m106 μm=1.0×10−5 m

Tube length

L=2 mm1 m1000 mm=2.0×10−3 m

Pressure drop, using 1 mmHg=133.322 Pa

P0−PL=60 mmHg−20 mmHg=40 mmHg

P0−PL=40 mmHg133.322 Pa1 mmHg=5332.88 Pa

## 2 Obtain the shear-stress distribution from the momentum balance

A momentum balance on the cylindrical shell in the figure relates the shear stress to the axial pressure gradient. Using Equation (6.38)

1rdrτrzdr=−dPdz

The pressure falls linearly from P0 to PL along the tube. Using Equations (6.40) and (6.42)

−dPdz=C1=P0−PLL

Multiply by r and integrate with respect to r

drτrzdr=P0−PLLr  rτrz=P0−PLLr22+C3

Divide by r. This gives Equation (6.43)

τrz=P0−PLLr2+C3r

Apply the boundary condition that the shear stress stays finite, and equals zero, on the centerline, τrz=0 at r=0. The term C3/r would be infinite at r=0 unless C3=0. This gives Equation (6.44)

τrz=P0−PL2Lr

The shear stress increases linearly from zero on the centerline to its largest value at the wall, r=R.

## 3 Calculate the maximum (wall) shear stress

Evaluate the distribution at r=R. Using Equation (6.45)

τw=τrzr=R=P0−PL2LR

Substitute the pressure drop, length and radius

τw=5332.88 Pa1.0×10−5 m22.0×10−3 m=0.0533288 Pa⋅m4.0×10−3 m=13.33 Pa

Convert to dyn/cm2, using 1 Pa=10 dyn/cm2

τmax=τw≈13.3 Pa=133 dyn/cm2 at the vessel wall, r=R=10 μm

The maximum shear stress acts on the arteriole wall, where r=R. The shear stress is zero on the centerline, r=0, and grows linearly with r in between.

## 4 Check the result

Units check

Pa⋅mm=Pa

Force-balance check: the net pressure force on the fluid cylinder equals the wall shear force on its surface, as in Equation (4.16)

P0−PLπR2=τw2πRL  τw=P0−PLR2L

The same expression results, so the wall shear stress depends only on the pressure drop and the tube geometry.

Role of the hematocrit: the shear-stress distribution comes from the momentum balance alone, before any constitutive relation is used. It therefore holds for Newtonian blood and for non-Newtonian models, such as the Casson model, alike. The hematocrit affects the apparent viscosity, and so the velocity profile, wall shear rate and flow rate, but it does not affect τw once P0−PL, L and R are specified.

Check against the source solution: the source’s result, 13.3 Pa at the wall, is correct. Its second method starts from the Newtonian parabolic profile, Equation (6.49), and uses Equation (6.46) to recover the same τw. Its closing note says that τw “follows directly from the parabolic solution”. The result does not actually need the parabolic, Newtonian profile: Equation (6.44) comes from the momentum balance alone, which is why the first method holds for any fluid.

Model note: 133 dyn/cm2 is high for an arteriole, where typical wall shear stresses are tens of dyn/cm2. The imposed pressure gradient, 40 mmHg over 2 mm, is large. In a 20 μm vessel, red cells are comparable in size to the lumen, so the continuum assumption is only approximate.