# Problem 2 — Casson Blood Flow in a Tube

Question

The constitutive relationship for flow of blood in a cylinder of radius R is

τrz1/2=τ01/2+ηdvzdr1/2 for τrz>τ0  dvzdr=0 for τrz≤τ0

- Show that the velocity profile is

rc<r<R: vz=ΔPR24ηL1−r2R2−83rc1/2R1/21−r3/2R3/2+2rcR1−rR

0<r<rc: vz=ΔPR24ηL1−83rc1/2R1/2+2rcR−rc23R2

where rc=2τ0L/ΔP is the critical radius, at which the magnitude of the shear stress equals the yield stress.

- Show that the ratio of the wall shear stress, τw=τrz(r=R), to the yield stress is

τwτ0=Rrc

Given: the Casson-type constitutive relation, with yield stress τ0 and Casson viscosity η; tube radius R, length L and pressure drop ΔP=P0−PL. No numerical values are given, so the solution stays symbolic.

Additional assumptions: steady, laminar, fully developed flow in a straight, rigid, horizontal tube; the flow is one-dimensional, vz=vz(r); no slip at the wall, vz(R)=0.

Form of the constitutive relation: the relation is used here in the Casson form of Equations (4.31) and (4.33), with η inside the square root

τrz1/2=τ01/2+−ηdvzdr1/2 for τrz>τ0  dvzdr=0 for τrz≤τ0

This corrects three points in the problem statement:

- The velocity decreases toward the wall, so dvz/dr<0. The square root must act on −dvz/dr, as in Equation (4.31).

- The target profile has the prefactor 1/η. That requires η inside the square root, so that S=η1/2 in Equation (4.31). With η outside the root, as the problem writes it, the prefactor would be 1/η2.

- The second condition should read τrz≤τ0, not τrz≤0.

## a Derive the velocity profile

### Shear-stress distribution

The shear-stress distribution in the tube follows from the momentum balance alone, for any fluid. Using Equation (6.43), with τrz finite on the centerline so that C3=0, this gives Equation (6.44)

τrz=ΔP2Lr

The shear stress is zero on the centerline and increases linearly to the wall.

### Critical radius

The fluid yields only where τrz>τ0. Set τrz(rc)=τ0

ΔP2Lrc=τ0  rc=2τ0LΔP

For r<rc, τrz<τ0 and the fluid moves as an unsheared plug. For rc<r<R, τrz>τ0 and the fluid shears.

It is convenient to write the yield stress in terms of the critical radius

τ0=ΔP rc2L

### Velocity gradient in the sheared region

Substitute the shear-stress distribution into the constitutive relation for rc<r<R

ΔP r2L1/2=τ01/2+−ηdvzdr1/2

Isolate the velocity-gradient term and square both sides

−ηdvzdr=ΔP r2L1/2−τ01/22

Expand the square

−dvzdr=1ηΔP r2L−2ΔP τ02L1/2r1/2+τ0

Replace τ0 with ΔP rc/2L. The middle coefficient becomes ΔP τ0/2L1/2=ΔP2rc/4L21/2=ΔP rc1/2/2L

−dvzdr=ΔP2ηLr−2rc1/2r1/2+rc

### Integrate with the no-slip condition

Integrate from a position r in the sheared region to the wall, where vz(R)=0

vz(R)−vz(r)=rRdvzdr′ dr′  vz(r)=rR−dvzdr′dr′

Substitute the gradient and integrate term by term

vz(r)=ΔP2ηLr′​22−43rc1/2r′​3/2+rcr′rR

Evaluate between the limits

vz(r)=ΔP2ηLR2−r22−43rc1/2R3/2−r3/2+rcR−r

Factor out R2/2, using R2−r2=R21−r2/R2, R3/2−r3/2=R3/21−r3/2/R3/2 and R−r=R1−r/R

vz(r)=ΔPR24ηL1−r2R2−83rc1/2R1/21−r3/2R3/2+2rcR1−rR

rc<r<R: vz=ΔPR24ηL1−r2R2−83rc1/2R1/21−r3/2R3/2+2rcR1−rR

### Velocity in the plug region

For 0<r<rc, τrz≤τ0. Using Equation (4.33)

dvzdr=0  vz(r)=vz(rc)

The plug velocity equals the sheared-region velocity at r=rc, because the velocity is continuous. Substitute r=rc into the sheared-region profile

vz(rc)=ΔPR24ηL1−rc2R2−83rc1/2R1/2+83rc1/2rc3/2R1/2R3/2+2rcR−2rc2R2

Combine the rc2/R2 terms: −1+83−2=−13

0<r<rc: vz=ΔPR24ηL1−83rc1/2R1/2+2rcR−rc23R2

The core, r<rc, moves as a solid plug at uniform velocity, because the shear stress there is below the yield stress. The shear stress in the plug is not zero. It still increases linearly with r, as Equation (6.44) requires, but it is too small to make the fluid yield.

### Check the profile

- No slip: at r=R, every bracketed term is zero, so vz(R)=0.

- Smooth join at rc: at r=rc, ΔP rc/2L1/2−τ01/2=0, so dvz/dr=0 on both sides of rc. The velocity and its slope are continuous.

- Newtonian limit: as τ0→0, rc→0 and the profile reduces to vz=ΔPR24ηL1−r2R2, the Poiseuille profile of Equation (6.49) with μ=η.

- No-flow limit: as rc→R, the plug velocity bracket becomes 1−83+2−13=0. Flow stops when the wall shear stress falls to the yield stress.

- Units: η has units of Pa⋅s, so ΔPR2/ηL has units of Pa⋅m2/Pa⋅s⋅m=m/s. The bracket is dimensionless.

## b Derive the wall-to-yield stress ratio

Evaluate the shear-stress distribution at the wall. Using Equation (6.45)

τw=τrz(R)=ΔP R2L

The yield stress, from the definition of the critical radius, is

τ0=ΔP rc2L

Divide the wall shear stress by the yield stress. The factor ΔP/2L cancels

τwτ0=ΔP R/2LΔP rc/2L

τwτ0=Rrc

The ratio of wall shear stress to yield stress equals the ratio of tube radius to plug radius. The plug fills a larger fraction of the tube as the wall shear stress approaches the yield stress, and flow stops (rc=R) when τw=τ0.

Check against the source solution: the final results match the target expressions, but the source has several errors:

- The constitutive relation needs −dvz/dr and η inside the square root, as noted above. When the source squares its relation, 1/η should become 1/η2 for the form it wrote. The target profile is correct only for the form with η inside the root.

- The source writes vz(R)−vz(r)=+1η∫[⋯]2dr. Because dvz/dr<0, the correct statement is vz(r)−vz(R)=1ηrR[⋯]2dr.

- The middle term of the source’s integral shows 43ΔPτ02Lr3/2. It should be 43ΔPτ02L1/2r3/2.

- The source says that in the plug “τrz is a constant”. In fact τrz still varies linearly with r there. It is the velocity that is constant, because τrz<τ0.

- The source writes the wall shear stress as ΔPR/rL. It should be ΔPR/2L, as in Equation (6.45).