# Problem — Blood Flow Around a Catheter

Question

Blood flows around a catheter placed within a vein in the body. The vein has D=1 cm and length L=5 cm. The radius of the catheter is εR=2 mm. The pressure drop from the inlet to the outlet of the blood vessel is ΔP=1 mmHg. The velocity of the blood around the catheter is

vz=ΔPR24μL1−r2R2−1−ε2lnεln​rR

Assume one-dimensional flow in the z-direction.

- Calculate the rate of flow of blood around the catheter

- What is the maximum shear rate, and where does it occur?

Blood flowing in the annulus between a catheter of radius εR and a vein of radius R

Given: D=1 cm, L=5 cm, εR=2 mm, ΔP=1 mmHg, and the velocity profile vz(r). The derivation of the profile is not required.

Additional assumptions:

- D=1 cm is the vein diameter, so the vein radius is R=D/2=0.5 cm. The problem calls D a radius, but the symbol D and the source solution both use R=0.5 cm. The Model note gives the result for R=1 cm.

- The blood viscosity is not given. The value μ=3.5 cP=0.0035 Pa⋅s, typical of whole blood at normal hematocrit and 37 ∘C, is assumed, as in the source. At the shear rates found in part (b), above 103 s−1, blood behaves nearly as a Newtonian fluid.

- The flow is steady, laminar and fully developed. The catheter is concentric with the vein and stationary, and both walls obey no slip.

## Convert the given values

Vein radius

R=D2=1 cm21 m100 cm=0.005 m

Catheter radius and radius ratio

εR=2 mm1 m1000 mm=0.002 m  ε=0.002 m0.005 m=0.40

Vein length

L=5 cm1 m100 cm=0.050 m

Pressure drop, using 1 mmHg=133.322 Pa

ΔP=1 mmHg133.322 Pa1 mmHg=133.322 Pa

## a Calculate the volumetric flow rate

The flow rate is the average velocity times the cross-sectional area, which is the integral of the velocity over the flow area. Using Equations (6.51) and (6.52)

QV=⟨v⟩A=A​vz dA

The flow occupies the annulus εR≤r≤R. A thin ring of radius r and thickness dr has area dA=2πr dr

QV=εRRvz(r) 2πr dr

Substitute the velocity profile. The constant B stands for the coefficient of the logarithm

QV=2πΔPR24μLεRR1−r2R2−Bln​rRr dr  B=1−ε2lnε

Change to the dimensionless radius ξ=r/R, so r=Rξ and dr=R dξ. The limits become ξ=ε and ξ=1

QV=2πΔPR44μLε1ξ−ξ3−B ξlnξdξ

Integrate the polynomial terms

ε1ξ−ξ3dξ=ξ22−ξ44ε1=14−ε22+ε44=1−ε224

Integrate the logarithmic term by parts, with u=lnξ, du=dξ/ξ, dv=ξ dξ and v=ξ2/2

∫ξlnξ dξ=ξ22lnξ−∫ξ22⋅dξξ=ξ22lnξ−ξ24

Evaluate between the limits, using ln1=0

ε1ξlnξ dξ=0−14−ε22lnε−ε24=−1−ε24−ε2lnε2

Combine the two results, and replace Blnε with 1−ε2

ε1ξ−ξ3−B ξlnξdξ=1−ε224+B1−ε24+ε21−ε22

Group the first and third terms, 1−ε241−ε2+2ε2=1−ε44, and substitute B in the second term

ε1ξ−ξ3−B ξlnξdξ=141−ε4+1−ε22lnε

Multiply by the prefactor to obtain the annular flow rate

QV=πΔPR48μL1−ε4+1−ε22lnε

The bracket is the correction to the Poiseuille flow rate of Equation (6.52). It equals 1 when ε→0, with no catheter.

Evaluate the bracketed correction factor with ε=0.40

ε2=0.16  ε4=0.0256  1−ε22=0.842=0.7056  ln0.40=−0.9163

1−ε4+1−ε22lnε=0.9744+0.7056−0.9163=0.9744−0.7701=0.2043

Evaluate the Poiseuille prefactor

πΔPR48μL=π133.322 Pa0.005 m480.0035 Pa⋅s0.050 m=2.618×10−7 Pa⋅m41.40×10−3 Pa⋅s⋅m=1.870×10−4 m3s

Multiply by the correction factor

QV=1.870×10−4 m3s0.2043=3.82×10−5 m3s

Convert, using 1 m3=106 mL and 1 min=60 s

QV=3.82×10−5 m3/s=38.2 mL/s≈2.3 L/min

The catheter reduces the flow to about 20% of the flow in the same vein without a catheter. Even though it fills only 16% of the cross-section, it adds a second no-slip surface in the fastest-moving part of the flow.

Check the average velocity over the annular area

⟨v⟩=QVπR21−ε2=3.82×10−5 m3/sπ0.005 m20.84=3.82×10−5 m3/s6.597×10−5 m2=0.579 m/s

## b Determine the maximum shear rate and its location

The shear rate is the velocity gradient normal to the flow. Using Equation (4.11) with n=r

γ=dvzdr

Write the profile with the catheter radius a=εR. Use R21−ε2=R2−a2 and lnε=−lnR/a

vz=ΔP4μLR2−r2−R2−a2lnR/aln​Rr

Differentiate with respect to r, using ddrln​Rr=−1r

dvzdr=ΔP4μL−2r+R2−a2rlnR/a

Differentiate once more to locate the extreme values

d2vzdr2=ΔP4μL−2−R2−a2r2lnR/a<0

The second derivative is negative everywhere, so dvz/dr decreases steadily from the catheter to the vein wall. The largest shear-rate magnitude must therefore occur at one of the two surfaces, r=a or r=R.

Calculate the common quantities

ΔP4μL=133.322 Pa40.0035 Pa⋅s0.050 m=1.905×105 m−1 s−1

R2−a2=0.005 m2−0.002 m2=2.5×10−5 m2−4.0×10−6 m2=2.10×10−5 m2

ln​Ra=ln​0.005 m0.002 m=ln2.5=0.9163

Shear rate at the vein wall, r=R=0.005 m

dvzdrR=1.905×105 m−1 s−1−20.005 m+2.10×10−5 m20.005 m0.9163

dvzdrR=1.905×105 m−1 s−1−0.010 m+0.004584 m=1.905×105 m−1 s−1−0.005416 m=−1.03×103 s−1

Shear rate at the catheter surface, r=a=0.002 m

dvzdra=1.905×105 m−1 s−1−20.002 m+2.10×10−5 m20.002 m0.9163

dvzdra=1.905×105 m−1 s−1−0.004 m+0.011459 m=1.905×105 m−1 s−10.007459 m=1.42×103 s−1

Compare the magnitudes, 1.42×103 s−1>1.03×103 s−1

γmax=1.42×103 s−1 at the catheter surface, r=εR=2 mm

The maximum shear rate occurs on the catheter surface, about 38% higher than at the vein wall. It is positive because the velocity increases moving outward from the catheter, and negative at the vein wall, where the velocity decreases toward zero.

The shear rate is zero where the velocity is greatest. Set dvz/dr=0

2r*=R2−a2r*lnR/a  r*=R2−a22lnR/a=2.10×10−5 m220.9163=3.39×10−3 m

The peak velocity lies at r*=3.39 mm, closer to the catheter (2 mm) than to the vein wall (5 mm). The velocity gradient is therefore steeper on the catheter side.

The corresponding shear stresses follow from Equation (6.46), τrz=−μ dvz/dr

τrzR=−0.0035 Pa⋅s−1.03×103 s−1=3.61 Pa  τrza=−0.0035 Pa⋅s1.42×103 s−1=−4.97 Pa

The largest shear-stress magnitude, 4.97 Pa, also acts on the catheter surface.

Check: the given profile satisfies both no-slip conditions. At r=R, ln1=0 and 1−r2/R2=0, so vz=0. At r=εR, the bracket becomes 1−ε2−1−ε2=0. The flow-rate units reduce as Pa⋅m4/Pa⋅s⋅m=m3/s, and the shear-rate units as m−1 s−1⋅m=s−1. For a blood density near 1050 kg/m3 (assumed), the Reynolds number based on the annular gap 2R1−ε=6 mm is about 1.0×103. This is below 2200, consistent with the laminar flow assumed.

Check against the source solution: the source’s numerical answers are correct: QV=38.2 mL/s and a maximum shear rate of 1.42×103 s−1 at the catheter. Four corrections:

- The problem calls D=1 cm the vein radius, but the source uses R=0.5 cm, treating D as the diameter.

- The viscosity is not given in the problem; the source assumes 3.5 cP.

- In the integration by parts, one line writes r4/4 where it should read r2/4.

- In the optional derivation, the momentum balance is written with −ΔP/L where Equation (6.38) gives +ΔP/L, and the Newtonian relation is used without the negative sign of Equation (6.46). The two sign errors cancel, so the final velocity profile is correct.

Model note: if the vein radius were literally R=1 cm, then ε=0.20 and the same method gives QV=1.27×10−3 m3/s (about 76 L/min), a mean velocity of 4.2 m/s and a Reynolds number near 2×104. That flow would be turbulent and physiologically unrealistic for a vein, which supports treating D=1 cm as the diameter.